- Purpose

  - The purpose of this document is to describe the procedure for assembling the scrub deck on a propane strip machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

E012327EQM0600BE1200 – Big Bertha XSM30 Propane Stripping Machine Pkg

E088398EQM0600BE1200 – Lil Bertha XSM24 Propane Stripping Machine Pkg

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

  - BB – Big Bertha

LB – Lil Bertha

- Procedure

  - Begin by placing the machine onto the pallet (E12697).  Center is a best as possible.  For a BB, place a piece of plywood under the rear wheels to prevent them from breaking through the skid.

  - Fasten 2 2x4’s around each of the rear wheels as shown.

  - Fasten a 2x4 on the front of the deck as shown.

  - Take a propane tank (88550) and place a yellow label (11663) in the location shown.

  - Remove the pins from the handle and rotate it to the transport position as shown.  Replace the pins into the locations shown to secure the handle.

  - Plane the propane tank onto the machine, strap it in place, and connect the fuel hose from the machine.

  - Place the manual packet into the top of the tank as shown.  This includes a manual, a warranty registration card, a tech service business card, and a Kawasaki engine manual (if available).  Removed the keys from the ignition switch and place them into the packet as well.

  - Place 2 metal bands over the machine in the locations shown.  Use cardboard pieces on the deck anywhere that the band will touch it.

  - Assemble a shipping box (12664) and place it over the machine.  Verify that the edges of the box fit over the sides of the pallet.

  - Staple the box to the pallet.

  - Place the machine ID and serial number barcode decal on 2 corners of the machine.

  - Place a “Do Not Top Load” label onto 2 opposite corners of the box.  Also place a UN3529 (RLB065A) sticker to the front.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 - 11/18/2013 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| Rev 02 - 1/18/2018 | Added Responsibilities Section and removed box cutting steps. |
| Rev 03 - 3/17/2025 | Updated steps, removed work center notes, added UN sticker |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________