- Purpose

  - The purpose of this document is to describe the procedure for assembling the knob on an orbital machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

E012801EQM0600BA1000 – Basic Sand Dragon, 20”

E012801EQM0607BE1200 – Crewman ORB 20

E012801EQM0607BA1200 – Basic Sand Dragon 20, Canadian

E012801EQM0607BE1200 – Crewman ORB 20, Canadian

E012806EQM0600BA1000 – Basic Sand Dragon, 20”, w/ Weights

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights

E012806EQM0607BA1200 – Basic Sand Dragon 20, w/Weights, Canadian

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights, Canadian

E012811EQM0600BA1000 – Basic Sand Dragon, 28”, w/Weights

E012811EQM0600BE1000 – Crewman 28 ORB, w/Weights

E012811EQM0607BA1200 – Basic Sand Dragon 28, w/Weights, Canadian

E012811EQM0607BE1200 – Crewman ORB 28, w/Weights Canadian

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

  - Pin assembly fixture

  - Snap ring pliers

- Definitions

- Procedure

  - Take 2 center bolts (89708) and place them in the assembly fixture as shown.  The pins in the nest go into the top holes on the center bolts.

  - Place the top nest in place.

  - Put a spring pin (89709) into each of the two holes shown.

  - Put the top plate into place.  The dowel pins go into the holes in the nest.

  - Put the assembly into a press and press the top plate until it makes contact with the nest.  This will fully insert the spring pins.

  - Move each center bolt down in the nest until the spring pin that was just pushed in locates itself in another hole.

  - Repeat the process.  Press 2 more spring pins (89709) into the holes shown.

  - Remove the center bolts from the nest.

  - Install a snap ring (89711) into the bottom groove on the center bolt as shown.

  - Slide a knob (89710) over the top end of the center bolt.  Install another snap ring (89711) into the top groove on the center bolt.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| 4/28/2015 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| 1/16/2018 | Added the Responsibilities Section |
| 10/15/2024 | Updated Machine ID’s |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________