- Purpose

  - The purpose of this document is to describe the procedure for assembling the pad plate on an orbital machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

E012801EQM0600BA1000 – Basic Sand Dragon, 20”

E012801EQM0607BE1200 – Crewman ORB 20

E012801EQM0607BA1200 – Basic Sand Dragon 20, Canadian

E012801EQM0607BE1200 – Crewman ORB 20, Canadian

E012806EQM0600BA1000 – Basic Sand Dragon, 20”, w/ Weights

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights

E012806EQM0607BA1200 – Basic Sand Dragon 20, w/Weights, Canadian

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights, Canadian

E012811EQM0600BA1000 – Basic Sand Dragon, 28”, w/Weights

E012811EQM0600BE1000 – Crewman 28 ORB, w/Weights

E012811EQM0607BA1200 – Basic Sand Dragon 28, w/Weights, Canadian

E012811EQM0607BE1200 – Crewman ORB 28, w/Weights Canadian

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Procedure

  - Take a pad driver backing plate (89612 for a 20” machine or 89655 for a 28” machine) and adhere a flex lock pad (89712 for a 20” machine or 89713 for a 28” machine) to the side shown.  There is no glue needed, just peel the back off of the flex lock.  Note the orientation of the backing plate.  The countersinks from the 8 center bolts should be up.  Keep the center punchouts from the flex lock pad.

  - Thread a barbed fitting (10080) into the plate as shown.

  - Take a bearing assembly (Refer to SOP EQ 083) and bolt it to the backing plate using 8 flat head screws (89865).  Put blue Loctite onto the threads before installing.

  - The dampeners now need to be fastened to the plate.  Place a nylon washer (89835) under a dampener (89656) and fasten it to the plate using a screw (89684).  Note which side of the plate it fastens to.

  - Repeat for the remaining locations.  A 20” machine requires 6 and a 28” machine requires 8.

  - Pad plate assembly is now complete.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| 4/28/2015 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| 1/16/2018 | Added the Responsibilities Section |
| 10/15/2024 | Updated Machine ID’s |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________