- Purpose

  - The purpose of this document is to describe the procedure for assembling the eccentric on an orbital machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

E012801EQM0600BA1000 – Basic Sand Dragon, 20”

E012801EQM0607BE1200 – Crewman ORB 20

E012801EQM0607BA1200 – Basic Sand Dragon 20, Canadian

E012801EQM0607BE1200 – Crewman ORB 20, Canadian

E012806EQM0600BA1000 – Basic Sand Dragon, 20”, w/ Weights

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights

E012806EQM0607BA1200 – Basic Sand Dragon 20, w/Weights, Canadian

E012806EQM0600BE1200 – Crewman ORB 20, w/Weights, Canadian

E012811EQM0600BA1000 – Basic Sand Dragon, 28”, w/Weights

E012811EQM0600BE1000 – Crewman 28 ORB, w/Weights

E012811EQM0607BA1200 – Basic Sand Dragon 28, w/Weights, Canadian

E012811EQM0607BE1200 – Crewman ORB 28, w/Weights Canadian

- Responsbilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

  - Hammer

- Definitions

- Procedure

  - Take an eccentric (89654) and install a roll pin (89915) into the hole shown.  This can be installed using a hammer.  Verify that the top of the roll pin is installed past the surface of the eccentric so the bearing will fit.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| 4/28/2015 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| 1/16/2018 | Added the Responsibilities Section |
| 10/15/2024 | Updated Machine ID’s |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________