- Purpose

  - The purpose of this document is to describe the procedure for assembling the Disk Engagement Plates for the Disk units.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following ID’s:

EP50323EQS0600BE1200 – 26” Disc Head Assembly

EP50324EQS0600BE1200 – 28” Disc Head Assembly

EP50325EQS0600BE1200 – 32” Disc Head Assembly

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Proceduref

  - Take a Disk Engagement Plate (E28589) and attach it to a Disk Alignment Coupler (E28588) using 6 Flat Head Screws (E29391).  Use blue Loctite on the screws.  Ensure proper orientation.  Torque bolts to a minimum of 6.3 ft-lbs.  Mark the bolt head for orientation with a paint marker.

  - Flip the assembly over and fasten a disk lock spring (E29518) by sandwiching it between a spring clip support (E29515) and a spring retaining bracket (E29519).  Attach it with 2 screws (E29390).  Use blue Loctite on the screws.  The spring should still be able to float.

  - Make a 2nd assembly as a mirror to the first by changing the location of the lock spring to the other side of the slot.  They should be a mirrored set.  Ensure all components are oriented properly.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| Rev 02 - 8/13/2021 | Added QC Check and torque check. |
| Rev 03 - 4/1/2025 | Reviewed and updated |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________