- Purpose

  - The purpose of this document is to describe the procedure for assembling the Recovery Tank on the Genesys Machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

EP50328EQS0600BE1200 – Recovery Tank Assembly (MSAS)

EP50290EQS0600BE1200 – Recovery Tank Assembly (G20)

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Procedure

  - Start with a recovery tank (E28575 – MSAS / E28886 – G20).

  - Tap out the hole where the lid mounts using a 1 ½” tap. Align tap with hole slowly thread tap through the hole. When tap is in halfway back the tap off making sure it is level.

  - Take a PVC tube (E29577) and but it into a 4” piece. Verify there are no obstructions inside the tank (Note: if obstruction is present grind out the area impeding the tube from being placed). Press the tube into a barbed fitting (E29531).  Thread the fitting into the appropriate location on the recovery tank.

  - Take a float ball cage mount (E29562) and stick a gasket (E28607) onto the bottom side.  Attach the mount to the tank using 2 hex bolts (E81089) and 2 washers (E28622).

Gasket goes between tank and Mount

  - Put a float ball shut off (E29563) onto the end of the mount.  Verify float balls are present. Attach it with a clamp (E28990).  Position interlocked portion of the clamp under the gooseneck.

  - Place the vac assembly (Refer to SOP 107 for assembly instructions) into the pocket on the tank.  -On the MSAS, place 3 spacers (E28850) between the bracket and the tank.  Then use 3 washers (E28622) and 3 hex bolts (E81072) to attach the bracket.

-On the G20, attach the vac bracket with 3 washers (E28622) and 3 hex bolts (E81072).

Spacers go between bracket and tank on the MSAS

  - Attach each of the 2 hinges (E28596) to the tank using 4 total flange bolts (E29569).  Slide the hinges towards the back of the tank as shown.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| TBD | This is the first version of the S.O.P. hence there are no revisions to this document. |
| 8/9/2021 | Remove the vac motor assembly portion. That is now its own SOP |
| 11/2/2023 | Updated steps for current process. |
| 3/31/2025 | Combined MSAS and G20 instructions. |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________