- Purpose

  - The purpose of this document is to describe the procedure for assembling the Vac Motor on the Genesys Machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

EP50328EQS0600BE1200 – Recovery Tank Assembly (MSAS)

EP50290EQS0600BE1200 – Recovery Tank Assembly (G20)

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Procedure

  - Take a vac motor (E28581-MSAS / E28993-G20) and place a gasket (E28608) onto it in the appropriate location.

  - Take 3 isolators (E29524 – MSAS / E28914 – G20) and put a straight line on each as shown.  Note: the isolators are slightly different between the two machines.  The photo is for reference only.

  - Place the stud end of the isolator into each of the 3 mounting holes on the vacuum bracket (E29523 – MSAS / E28995 – G20).  Attach the isolators with a hex nut (E29565).  Hold the isolators with vise grips or channel locks to prevent them from spinning.  Tighten the nuts with a nut driver or non-impact tool.

  - Place the vac mount bracket with the isolators onto the vac motor in the proper orientation.  On the MSAS, attach it with 3 more hex nuts (E29565).  For the G20, attach it with 3 bolts (10734).  Hold the isolators with vise grips or channel locks to prevent them from spinning.  Tighten the nuts with a nut driver or non-impact tool.

G20

MSAS

  - Ensure that the nuts are not tightened so much that the isolators twist.  This will cause a failure.  Make sure the lines that were put on them are straight after assembly is complete.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| Rev 02 - 3/31/2025 | Combined MSAS and G20 Instructions. Updated accordingly |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________