- Purpose

  - The purpose of this document is to describe the procedure for assembling the Inner Electrical Panel on the Genesys 20 Machine.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machine ID’s:

EP50343EQS0600BE1200 – Inner Electrical Panel, G20 TR Disk

EP50344EQS0600BE1200 – Inner Electrical Panel, G20 TR ORB

EP50345EQS0600BE1200 – Inner Electrical Panel, G20 PA

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Procedure

  - Start with an Inner Electrical Panel (E28891).

  - Add red Loctite onto the standoff threads.

  - Attach an electrical standoff (E28950) to each stud and snug it in place.  Be careful to not overtighten.  Put some red Loctite onto a set screw (E29447) and thread it into the standoff.  Repeat for the other standoff.

  - Place a spacer (E29373) over each of the 5 studs where the control board mounts.

  - Attach a control board (E28892 for a Transaxle Units / E28810 for a pad assist unit) onto the 5 studs using 4 lock nuts (E29381).  Do not put a nut on the center stud.

Don’t put a nut on center stud

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| TBD | This is the first version of the S.O.P. hence there are no revisions to this document. |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________