- Purpose

  - The purpose of this document is to describe the procedure for assembling a propane burnisher pad driver.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following sub-assembly’s:

EP50040EQS0600BE1200 – Flex lok assembly 27” complete

EP50027EQS0600BE1200 – Flex lok assembly 24” complete

EP50028EQS0600BE1200 – Flex lok assembly 21” complete

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the sub-assembly work order.

  - Hand/Power Tools

  - Riveter

- Definitions

- Procedure

  - Start by bolting the shaft plate (10586) to the flex-lok (10252 for 27”, 10251 for 24”, or 10250 for 21”) using 6 bolts (10112), lock washers (81067), flat washers (12444), and nuts (81063).

Nut

Lock Washer

Flat Washer

Flex-Lok

Nut

  - Unscrew the top and bottom pieces of the center-lock.  Attach the appropriate half of the center lock (12517) to the flex-lok using 3 rivets (10850) and the pneumatic riveter.  Use the three holes shown in the picture below.

Rivet

Rivet

Rivet

Center Lock

  - Place a burnishing pad (84627 for 27”, 84624 for 24”, or 84621 for 21”) onto the gripping side of the flex-lok.  Center the pad as best as possible.

Burnishing Pad

  - Thread the remaining half of the center lock into the center of the pad driver.  Thread it until the 2 halves start to ratchet together.

Center-Lock

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 - 11/18/2013 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| Rev 02 - 1/12/2018 | Added the Responsibilities section |
| Rev 03 - 3/21/2025 | Removed work center note |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________