- Purpose

  - The purpose of this document is to describe the procedure for load testing a propane machine battery.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machines:

E012800EQM0600BE1200 – 21” Optima XR

E012805EQM0600BE1200 – 21” Optima XR w/ Clutch

E012815EQM0600BE1200 – 21” Optima XR w/ Emissions & Clutch

E012830EQM0600BE1200 – 27” Optima XR

E012835EQM0600BE1200 – 27” Optima XR w/ Clutch

E012840EQM0600BE1200 – 27” Optima XR w/ Emissions

E012850EQM0600BE1200 – 27” Optima XR w/ Dust Control

E012860EQM0600BE1200 – 27” Optima XR w/ Clutch & Dust Control

E012865EQM0600BE1200 – 27” Optima XR w/ Emissions & Dust Control

E012875EQM0600BE1200 – 27” Optima XR w/ Emissions, Clutch, & Dust Control

E012327EQM0600BE1200 – Big Bertha XSM30 Propane Stripping Machine Pkg

E088398EQM0600BE1200 – Lil Bertha XSM24 Propane Stripping Machine Package

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

  - Battery Load Tester

- Definitions

- Procedure

  - Take a battery (89827) and clamp the terminals using the appropriate clamps on the load tester.

  - Verify that the voltage reading is acceptable.  It should read 12V or higher.

  - Press and hold the red button for a couple seconds until the needle stops moving.

  - Verify that the needle stays in the green area at the 200CCA line.

Needle should stay above this line

  - If the battery falls below, it is no good.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 - 11/22/2013 | This is the first version of the S.O.P. hence there are no revisions to this document. |
| Rev 02 - 1/12/2018 | Added responsibilities section |
| Rev 03 - 5/4/2018 | Updated for the Optima XR upgrades |
| Rev 04 - 3/21/2025 | Removed work center notes and updated machine ID list in Scope 2.01 |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________