- Purpose

  - The purpose of this document is to describe the procedure for assembling an Optima Propane Burnisher front bearing.

- Scope

  - This SOP applies to all equipment assembly team members.  The process applies to the following machines:

E012800EQM0600BE1200 – 21” Optima XR

E012805EQM0600BE1200 – 21” Optima XR w/ Clutch

E012815EQM0600BE1200 – 21” Optima XR w/ Emissions & Clutch

E012830EQM0600BE1200 – 27” Optima XR

E012835EQM0600BE1200 – 27” Optima XR w/ Clutch

E012840EQM0600BE1200 – 27” Optima XR w/ Emissions

E012850EQM0600BE1200 – 27” Optima XR w/ Dust Control

E012860EQM0600BE1200 – 27” Optima XR w/ Clutch & Dust Control

E012865EQM0600BE1200 – 27” Optima XR w/ Emissions & Dust Control

E012875EQM0600BE1200 – 27” Optima XR w/ Emissions, Clutch, & Dust Control

- Responsibilities

  - The operations manager/supervisor is responsible for enforcing this SOP.

- Materials

  - The proper materials needed for this process are listed on the machine work order.

- Definitions

- Procedure

  - Start with a drive shaft hub (E80131).  Press a bearing (E81771) into each side of the hub using the press and the steel block.

Bottom Side

Top Side

  - Install the shaft (E80132) into the hub/bearings in the orientation shown.  The shoulder on the shaft should stop at the bearing.  You can use the press to do this.

  - Install a snap ring (E12976) into the groove on the bottom of the shaft to hold it in place.

- References (optional section as needed)

- Training Guide

Functional Training Groups Requiring Training

Equipment Assembly Group

- SOP Change Control

| Date of Change | Reason for Change |
| --- | --- |
| Rev 01 – 11/18/2025 | This is the first version of the S.O.P. hence there are no revisions to this document. |

Approvals

Submitter/Author Approval:__________________________________ Date:______________________

Management Approval:______________________________________ Date:______________________

QAU Approval:_____________________________________________ Date:______________________